Tender specification - AAP13 1.5 LI-LI, Modular distribution terminals, PUSH IN, 1,5 mm², 250 V, 16 A, dark beige

Manufracturer:	Weidmüller Interface GmbH & Co KG

Brand:	Klippon®Connect

Product line:	A-series

Type:	AAP13 1.5 LI-LI

Order No.:	1988280000

Rating data

Rated cross-section:	1,5 mm²

Rated voltage:	250 V

Rated impulse withstand voltage:	4 kV

Rated current:	16 A

Power loss:	0,47 W

Standards:	IEC/EN 60947-7-1:2009

Current at maximum wires:	16 A

UL rating data

Voltage size C (cURUS):	150 V

Current size C (CURUS):	13 A

CSA rating data

Voltage size C (CSA):	150 V

Current size C (CSA):	13 A

Rating data ATEX

Voltage (ATEX):	220 V

Current (ATEX):	13 A

Rating data IECEX

Voltage max. (IECEX):	220 V

Current (IECEX):	13 A

Conductors for clamping (rated connection)

Wire connection cross section AWG, min.:	AWG 14

Wire connection cross section AWG, max.:	AWG 26

Wire connection cross-section, solid core, min.:	0,5 mm²

Wire connection cross-section, solid core, max.:	1,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, max.:	1,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, max.:	1,5 mm²

Wire connection cross-section, finely stranded min.:	0,5 mm²

Wire connection cross-section, finely stranded, max.:	1,5 mm²

Wire connection cross-section, stranded, min.:	0,5 mm²

Wire connection cross-section, stranded, max.:	1,5 mm²

Material data

Material:	Wemid

Colour:	dark beige

UL 94 flammability rating:	V-0

Temperatures:

Continuous operating temp., min.:	-60 °C

Continuous operating temp., max.:	130 °C