Tender specification - A2C 2.5 OR, Feed-through terminal, PUSH IN, 2,5 mm², 800 V, 24 A, orange

Manufracturer:	Weidmüller Interface GmbH & Co KG

Brand:	Klippon®Connect

Product line:	A-series

Type:	A2C 2.5 OR

Order No.:	1521930000

Rating data

Rated cross-section:	2,5 mm²

Rated voltage:	800 V

Rated impulse withstand voltage:	8 kV

Rated current:	24 A

Power loss:	0,77 W

Standards:	IEC 60947-7-1

Current at maximum wires:	24 A

UL rating data

Voltage size C (cURUS):	600 V

Current size C (CURUS):	20 A

CSA rating data

Voltage size C (CSA):	600 V

Current size C (CSA):	20 A

Rating data ATEX

Voltage (ATEX):	550 V

Current (ATEX):	20 A

Rating data IECEX

Voltage max. (IECEX):	550 V

Current (IECEX):	20 A

Conductors for clamping (rated connection)

Wire connection cross section AWG, min.:	AWG 12

Wire connection cross section AWG, max.:	AWG 28

Wire connection cross-section, solid core, min.:	0,5 mm²

Wire connection cross-section, solid core, max.:	2,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, max.:	2,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, max.:	2,5 mm²

Wire connection cross-section, finely stranded min.:	0,5 mm²

Wire connection cross-section, finely stranded, max.:	2,5 mm²

Wire connection cross-section, stranded, min.:	0,5 mm²

Wire connection cross-section, stranded, max.:	2,5 mm²

Material data

Material:	Wemid

Colour:	orange

UL 94 flammability rating:	V-0

Temperatures:

Continuous operating temp., min.:	-60 °C

Continuous operating temp., max.:	130 °C