Tender specification - ALO 6 BL, Supply terminal, PUSH IN, 6 mm², 800 V, 41 A, blue

Manufracturer:	Weidmüller Interface GmbH & Co KG

Brand:	Klippon®Connect

Product line:	A-series

Type:	ALO 6 BL

Order No.:	2065120000

Rating data

Rated cross-section:	6 mm²

Rated voltage:	800 V

Rated impulse withstand voltage:	8 kV

Rated current:	41 A

Power loss:	1,31 W

Standards:	IEC 60947-7-1

Current at maximum wires:	41 A

UL rating data

Voltage size C (cURUS):	600 V

Current size C (CURUS):	38 A

CSA rating data

Voltage size C (CSA):	600 V

Current size C (CSA):	38 A

Rating data ATEX

Voltage (ATEX):	550 V

Current (ATEX):	33 A

Rating data IECEX

Voltage max. (IECEX):	550 V

Current (IECEX):	33 A

Conductors for clamping (rated connection)

Wire connection cross section AWG, min.:	AWG 8

Wire connection cross section AWG, max.:	AWG 22

Wire connection cross-section, solid core, min.:	0,5 mm²

Wire connection cross-section, solid core, max.:	6 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, max.:	6 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, max.:	6 mm²

Wire connection cross-section, finely stranded min.:	0,5 mm²

Wire connection cross-section, finely stranded, max.:	6 mm²

Wire connection cross-section, stranded, min.:	0,5 mm²

Wire connection cross-section, stranded, max.:	6 mm²

Material data

Material:	Wemid

Colour:	blue

UL 94 flammability rating:	V-0

Temperatures:

Continuous operating temp., min.:	-60 °C

Continuous operating temp., max.:	130 °C