Tender specification - A4C 1.5 LTGY, Feed-through terminal, PUSH IN, 1,5 mm², 500 V, 17,5 A, light grey

Manufracturer:	Weidmüller Interface GmbH & Co KG

Brand:	Klippon®Connect

Product line:	A-series

Type:	A4C 1.5 LTGY

Order No.:	2534420000

Rating data

Rated cross-section:	1,5 mm²

Rated voltage:	500 V

Rated impulse withstand voltage:	6 kV

Rated current:	17,5 A

Power loss:	0,56 W

Standards:	IEC 60947-7-1

Current at maximum wires:	17,5 A

UL rating data

Voltage size C (cURUS):	300 V

Current size C (CURUS):	13 A

CSA rating data

Voltage size C (CSA):	300 V

Current size C (CSA):	13 A

Rating data ATEX

Voltage (ATEX):	440 V

Current (ATEX):	15 A

Rating data IECEX

Voltage max. (IECEX):	440 V

Current (IECEX):	15 A

Conductors for clamping (rated connection)

Wire connection cross section AWG, min.:	AWG 14

Wire connection cross section AWG, max.:	AWG 26

Wire connection cross-section, solid core, min.:	0,5 mm²

Wire connection cross-section, solid core, max.:	1,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/1, max.:	1,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, min.:	0,5 mm²

Wire connection cross-section, finely stranded with

wire-end ferrules DIN 46228/4, max.:	1,5 mm²

Wire connection cross-section, finely stranded min.:	0,5 mm²

Wire connection cross-section, finely stranded, max.:	1,5 mm²

Wire connection cross-section, stranded, min.:	0,5 mm²

Wire connection cross-section, stranded, max.:	1,5 mm²

Material data

Material:	Wemid

Colour:	light grey

UL 94 flammability rating:	V-0

Temperatures:

Continuous operating temp., min.:	-60 °C

Continuous operating temp., max.:	130 °C